The State of Texas §                                                                                                                              Live Oak Partners, L.P.

County of McMullen §                                                                                                                                               25.52 Acres

Description of 25.52 acres, more or less, being wholly within the Michael Hely Survey, Abstract No. 6, and being part of Block Nos. 10 and 11 of the Subdivision of the J. T. Calliham Ranch, recorded in Volume 1, Page 5, of the Plat Records of McMullen County, Texas, said 25.52 acres being a remaining portion of a called 100.52 acres conveyed to Live Oak Partners, L.P. by General Warranty Deed dated August 30th, 2011, recorded in Volume 13, Page 150 et seq, of the Official Public Records of McMullen County, Texas, said 25.52 acres having a bearing basis derived from GPS observations in the Texas State Plane Coordinate System, NAD 1983 South Central Zone, made on May 31st , 2011, with all other bearings relative thereto;

Beginning at a ½” iron pin with plastic cap stamped “Little 6163” found on the southwest boundary of said 100.52 acres, being on the southwest boundary of said Block No. 10, being on the northeast boundary of a called 194.571 acres conveyed to Rick Day Cattle Co., LLP by General Warranty Deed with Vendor’s Lien dated July 28th, 2005, recorded in Volume 437, Page 467 et seq, of the Deed Records of McMullen County, being on the northeast boundary of Block No. 9 of said Subdivision, being the southwest corner of a called 25.00 acres conveyed to Knox Oil Field Supply, Inc. by General Warranty Deed with Vendor’s Lien dated November 30th, 2011, recorded in Volume 21, Page 386 et seq, of the Official Public Records of McMullen County, Texas, for the southeast corner and Point of Beginning of this tract;  from Whence an 2” pipe corner fence post found for the southeast corner of said 100.52 acres, being the northeast corner of said 194.571 acres, bears South 71° 56’ 49” East, a distance of 778.85’;

Thence North 71º 56’ 49” West, (Record), (South 71º 57’ 23” East (Adjoiner Record)), with the northeast boundary of said 194.571 acres, with the northeast boundary of said Block No. 9, with the southwest boundary of said 100.52 acres, with the southwest boundary of said Block No. 10, and generally along a fence, a distance of 974.34’, to a ½” iron pin with plastic cap stamped “Little 6163” found on the northeast boundary of said 194.571 acres, being on the northeast boundary of said Block No. 9, being on the southwest boundary of said 100.52 acres, being on the southwest boundary of said Block No. 10,  being the southeast corner of a called 50.00 acres conveyed to Missouri Basin Well Service, Inc. by Special Warranty Deed dated February 16, 2012, recorded in Volume 23, Page 250 et seq, of the Official Public Records of McMullen County, Texas, for the southwest corner of this tract; from Whence a ½” iron pin with plastic cap stamped “Little 6163” found for the southwest corner of said 100.52 acres, being the northwest corner of said 194.571 acres, bears North 71° 56’ 49” West, a distance of 2675.82’;

Thence North 18º 13’ 35” East, crossing and passing through said 100.52 acres, and crossing and passing through said Block Nos. 10 and 11, and with the southeast boundary of said 50.00 acres, a distance of 1013.33’, to a ½” iron pin with plastic cap stamped “Little 6163” found on the south right-of-way of State Highway No. 72, being on the north boundary of said 100.52 acres, being the northeast corner of said 50.00 acres, for the northwest corner of this tract; from Whence a Type I Concrete Highway Monument found, bears North 85° 31’ 25” West, a distance of 103.64’;

Thence South 85º 31’ 25” East, (Record), with the south right-of-way of State Highway No. 72, with the north boundary of said 100.52 acres, a distance of 399.71’, to a Type I Concrete Highway Monument found on the south right-of-way of State Highway No. 72, being an interior angle corner of said 100.52 acres, for an interior angle corner of this tract;

Thence South 88º 27’ 25” East, (Record), with the south right-of-way of State Highway No. 72, with the north boundary of said 100.52 acres, a distance of 379.08’, (Record), to a Type I Concrete Highway Monument found on the south right-of-way of State Highway No. 72, being an interior angle corner of said 100.52 acres, for an interior angle corner of this tract;

Thence South 88º 39’ 10” East, (Record), with the south right-of-way of State Highway No. 72, with the north boundary of said 100.52 acres, a distance of 233.00’, to a ½” iron pin with plastic cap stamped “Little 6163” found on the south right-of-way of State Highway No. 72, being on the north boundary of said 100.52 acres, being the northwest corner of said 25.00 acres, for the northeast corner of this tract;

Thence South 18º 13’ 35” West, crossing and passing through said 100.52 acres, and crossing and passing through said Block Nos. 10 and 11, and with the northwest boundary of said 25.00 acres, a distance of 1281.87’, to the Point of Beginning.

I, Stacy L. Little, Registered Professional Land Surveyor, do hereby certify that this description was prepared from an actual survey made on the ground, under my supervision.  This the 1st day of March, 2012.  This description is to be attached and made a part of the accompanying plat.

______________________________________

Stacy L. Little, R.P.L.S.

Texas Registration No. 6163