Assistance Request Information

for

On-Site Septic Tank Absorption Field Sewage Disposal System

Date of assistance

6/19/24

Person(s) requesting

Kevin Presnell c/o Farm Flex Capital PO Box 123 Morgantown, IN 46160

Telephone number

| 317 | 752 - 6880 |
| --- | --- |

Name of owner

Site location

14797 N State Road 156, Patriot, IN 47038

Legal land description

SW ¼ of the NW ¼ section 25, T. 3 N. and R. 1 W.

Parcel ID

78-02-25-200-010.000-006

| Soil survey | Switzerland |  |
| --- | --- | --- |
| Atlas sheet | 7 |  |

Site geology

Loess over Ordovician bedrock residuum

Purpose of assistance

Residential Septic Evaluation

| Number of bedrooms | 3 |
| --- | --- |

Persons present during investigation

Dena L. Anderson, Kevin Presnell, Joe Parham

Stage of development

Preliminary to rezoning and new construction

Location of well or water source

Rural water is the source for these building lots

Comments

This report contains preliminary investigations for lot sizing and feasibility. One small backhoe pit was conducted in the river front lots in the area most likely to support a septic system. The ridge lots were hand auger/push probe evaluated. Upon sale and prior to home construction a full site evaluation will be needed to meet state septic system evaluation minimums and to accommodate furture home placement

Given my present expertise of Soil Science, Soil Genesis, Soil Morphology, Glacial Geology, and Geomorphology in accordance with Federal, State, and Local regulations in addition to the information provided to me by the client, landowner, real estate agent, or county sanitarian, I Dena L. Anderson certify, to the best of my knowledge, that the information contained in this report is complete and accurate.  The results of this on-site soil investigation reflect the soil properties, genesis, landscape evolution, and chemical data of the indicated boring locations.  Investigations in other areas will reflect different results.  This report does not pertain if there are any changes or alterations as a result of grading, filling, compacting, or any other disturbances that would otherwise destroy the soil properties described at the time this on-site investigation was completed.  Duplication or alteration of this report by any party other than Glacier’s Edge Soil Consulting renders this report invalid.  This report is not valid unless signed by Dena L. Anderson.

/s/ Dena L. Anderson		                7/16/24

|  | Dena L. Anderson |  | Date |
| --- | --- | --- | --- |
|  | Professional Soil Scientist |  |  |
|  | IRSS - #90 IAPSC - CPSC #49 |  |  |

| BORING #: |  |  |  | 1 |  |  |  |  | DESCRIPTION OF BORING LOCATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Small backhoe pit near center of the proposed lot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upland |  | Terrace |  |  |  |  |  | Outwash Plain |  |  |  |  |  | Ridge Top |  |  |  | Upper 1/3 |  |  | Middle |  |  |  |  | Lower 1/3 |  |  |  | Pothole |  |  |  | Depression |  |  | Broad Flat |  |  | Flood Plain |  |
| X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ponding or Flooding: |  |  |  |  |  |  | NO |  |  | % Slope |  |  | 8 |  |  | Aspect |  |  | N |  |  |  | Family Classification: |  |  |  |  |  | Fine, illitic, mesic Typic Hapludalfs |  |  |  |  |  |  |  |  |  |  |  |  |
| < 0.25 gpd/ft2 |  |  | > 1.20 gpd/ft2 |  |  |  |  |  |  | Seasonal High Water Table |  |  |  |  |  |  | Fragipan |  |  |  |  | Glacial Till |  |  |  |  | Bedrock |  |  |  | Massive Clay |  |  |  | Poor Filter |  |  |  | Other |  |  |
|  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vegetation at site: |  |  |  |  | Idle/scattered trees |  |  |  |  |  |  | Disturbance? |  |  |  |  |  | NO |  | Soil Series mapped: |  |  |  |  |  | Pate |  |  |  |  |  |  | Soil Series of Boring: |  |  |  |  | Pate |  |  |  |
| SOIL PROPERTIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DEPTH | SOIL COLOR |  |  |  |  |  |  |  |  |  |  |  |  |  | TEXTURE |  | STRUCTURE |  |  |  |  | CONSIS- |  |  | ROOTS |  |  | EFFERVES- |  |  |  | PARENT |  |  |  | % |  | OTHER |  |  |  |
| (IN) | MATRIX |  |  |  |  | MOTTLES |  |  |  |  | COATINGS |  |  |  | USDA |  | USDA |  |  |  |  | TANCY |  |  |  |  |  | CENCE |  |  |  | MATERIAL |  |  |  | CLAY |  |  |  |  |  |
| 0 – 7 | 10YR 4/2 |  |  |  |  |  |  |  |  |  |  |  |  |  | Silt loam |  | Moderate medium granular |  |  |  |  | Friable |  |  | Many |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 24 |  | 3% channers |  |  |  |
| A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 – 38 | 10YR 5/4 |  |  |  |  |  |  |  |  |  | 10YR 5/3 Clay |  |  |  | Silty clay loam |  | Moderate fine and medium subangular blocky |  |  |  |  | Firm |  |  | Many to few in the lower part |  |  |  |  |  |  | Limestone/ Colluvium |  |  |  | 38 |  | 5% channers |  |  |  |
| 2Bt1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 38 – 60 | 2.5Y 5/3 |  |  |  |  |  |  |  |  |  | 10YR 4/2 Clay |  |  |  | Silty clay |  | Weak medium subangular blocky |  |  |  |  | Firm |  |  | Few |  |  |  |  |  |  | Shale residuum |  |  |  | 52 |  | 8% channers |  |  |  |
| 3Bt2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| BORING #: |  |  |  | 2 |  |  |  |  | DESCRIPTION OF BORING LOCATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Small backhoe pit near center of the proposed lot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upland |  | Terrace |  |  |  |  |  | Outwash Plain |  |  |  |  |  | Ridge Top |  |  |  | Upper 1/3 |  |  | Middle |  |  |  |  | Lower 1/3 |  |  |  | Pothole |  |  |  | Depression |  |  | Broad Flat |  |  | Flood Plain |  |
| X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ponding or Flooding: |  |  |  |  |  |  | NO |  |  | % Slope |  |  | 8 |  |  | Aspect |  |  | N |  |  |  | Family Classification: |  |  |  |  |  | Fine, illitic, mesic Typic Hapludalfs |  |  |  |  |  |  |  |  |  |  |  |  |
| < 0.25 gpd/ft2 |  |  | > 1.20 gpd/ft2 |  |  |  |  |  |  | Seasonal High Water Table |  |  |  |  |  |  | Fragipan |  |  |  |  | Glacial Till |  |  |  |  | Bedrock |  |  |  | Massive Clay |  |  |  | Poor Filter |  |  |  | Other |  |  |
|  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vegetation at site: |  |  |  |  | Idle/scattered trees |  |  |  |  |  |  | Disturbance? |  |  |  |  |  | NO |  | Soil Series mapped: |  |  |  |  |  | Pate |  |  |  |  |  |  | Soil Series of Boring: |  |  |  |  | Pate |  |  |  |
| SOIL PROPERTIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DEPTH | SOIL COLOR |  |  |  |  |  |  |  |  |  |  |  |  |  | TEXTURE |  | STRUCTURE |  |  |  |  | CONSIS- |  |  | ROOTS |  |  | EFFERVES- |  |  |  | PARENT |  |  |  | % |  | OTHER |  |  |  |
| (IN) | MATRIX |  |  |  |  | MOTTLES |  |  |  |  | COATINGS |  |  |  | USDA |  | USDA |  |  |  |  | TANCY |  |  |  |  |  | CENCE |  |  |  | MATERIAL |  |  |  | CLAY |  |  |  |  |  |
| 0 – 6 | 10YR 4/2 |  |  |  |  |  |  |  |  |  |  |  |  |  | Silt loam |  | Moderate medium granular |  |  |  |  | Friable |  |  | Many |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 24 |  | 3% channers |  |  |  |
| A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 – 41 | 10YR 5/4 |  |  |  |  |  |  |  |  |  | 10YR 5/3 Clay |  |  |  | Silty clay loam |  | Moderate fine and medium subangular blocky |  |  |  |  | Firm |  |  | Many to few in the lower part |  |  |  |  |  |  | Limestone/ Colluvium |  |  |  | 30 |  | 5% channers |  |  |  |
| 2Bt1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 41 – 60+ | 2.5Y 6/4 |  |  |  |  |  |  |  |  |  | 10YR 6/3 Clay |  |  |  | Silty clay |  | Weak medium subangular blocky |  |  |  |  | Firm |  |  | Few |  |  |  |  |  |  | Shale residuum |  |  |  | 56 |  | 8% channers |  |  |  |
| 3Bt2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| BORING #: |  |  |  | 3 |  |  |  |  | DESCRIPTION OF BORING LOCATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Small backhoe pit near center of the proposed lot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upland |  | Terrace |  |  |  |  |  | Outwash Plain |  |  |  |  |  | Ridge Top |  |  |  | Upper 1/3 |  |  | Middle |  |  |  |  | Lower 1/3 |  |  |  | Pothole |  |  |  | Depression |  |  | Broad Flat |  |  | Flood Plain |  |
| X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ponding or Flooding: |  |  |  |  |  |  | NO |  |  | % Slope |  |  | 7 |  |  | Aspect |  |  | N |  |  |  | Family Classification: |  |  |  |  |  | Fine, illitic, mesic Typic Hapludalfs |  |  |  |  |  |  |  |  |  |  |  |  |
| < 0.25 gpd/ft2 |  |  | > 1.20 gpd/ft2 |  |  |  |  |  |  | Seasonal High Water Table |  |  |  |  |  |  | Fragipan |  |  |  |  | Glacial Till |  |  |  |  | Bedrock |  |  |  | Massive Clay |  |  |  | Poor Filter |  |  |  | Other |  |  |
|  |  |  |  |  |  |  |  |  |  | +58” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | +58” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vegetation at site: |  |  |  |  | Idle/scattered trees |  |  |  |  |  |  | Disturbance? |  |  |  |  |  | NO |  | Soil Series mapped: |  |  |  |  |  | Pate |  |  |  |  |  |  | Soil Series of Boring: |  |  |  |  | Pate |  |  |  |
| SOIL PROPERTIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DEPTH | SOIL COLOR |  |  |  |  |  |  |  |  |  |  |  |  |  | TEXTURE |  | STRUCTURE |  |  |  |  | CONSIS- |  |  | ROOTS |  |  | EFFERVES- |  |  |  | PARENT |  |  |  | % |  | OTHER |  |  |  |
| (IN) | MATRIX |  |  |  |  | MOTTLES |  |  |  |  | COATINGS |  |  |  | USDA |  | USDA |  |  |  |  | TANCY |  |  |  |  |  | CENCE |  |  |  | MATERIAL |  |  |  | CLAY |  |  |  |  |  |
| 0 – 4 | 10YR 4/2 |  |  |  |  |  |  |  |  |  |  |  |  |  | Silt loam |  | Moderate medium granular |  |  |  |  | Friable |  |  | Many |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 24 |  | 3% channers |  |  |  |
| A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 – 28 | 10YR 5/4 |  |  |  |  |  |  |  |  |  | 10YR 5/3 Clay |  |  |  | Silty clay loam |  | Moderate fine and medium subangular blocky |  |  |  |  | Firm |  |  | Many to few in the lower part |  |  |  |  |  |  | Limestone/ Colluvium |  |  |  | 30 |  | 5% channers |  |  |  |
| 2Bt1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 – 48 | 2.5Y 5/4 |  |  |  |  |  |  |  |  |  | 10YR 5/3 Clay |  |  |  | Silty clay |  | Weak medium subangular blocky |  |  |  |  | Firm |  |  | Few |  |  |  |  |  |  | Shale residuum/ Colluvium |  |  |  | 47 |  | 8% channers |  |  |  |
| 3Bt2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 48 – 58+ | 2.5Y 6/4 |  |  |  |  |  |  |  |  |  | 10YR 6/3 Clay |  |  |  | Channery Silty clay |  | Weak medium subangular blocky |  |  |  |  | Firm |  |  |  |  |  |  |  |  |  | Shale residuum/ Colluvium |  |  |  | 56 |  | 28% channers |  |  |  |
| 3Bt3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| BORING #: |  |  |  | 4 |  |  |  |  | DESCRIPTION OF BORING LOCATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Small backhoe pit near center of the proposed lot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upland |  | Terrace |  |  |  |  |  | Outwash Plain |  |  |  |  |  | Ridge Top |  |  |  | Upper 1/3 |  |  | Middle |  |  |  |  | Lower 1/3 |  |  |  | Pothole |  |  |  | Depression |  |  | Broad Flat |  |  | Flood Plain |  |
| X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ponding or Flooding: |  |  |  |  |  |  | NO |  |  | % Slope |  |  | 7 |  |  | Aspect |  |  | N |  |  |  | Family Classification: |  |  |  |  |  | Fine, illitic, mesic Typic Hapludalfs |  |  |  |  |  |  |  |  |  |  |  |  |
| < 0.25 gpd/ft2 |  |  | > 1.20 gpd/ft2 |  |  |  |  |  |  | Seasonal High Water Table |  |  |  |  |  |  | Fragipan |  |  |  |  | Glacial Till |  |  |  |  | Bedrock |  |  |  | Massive Clay |  |  |  | Poor Filter |  |  |  | Other |  |  |
|  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | +58” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vegetation at site: |  |  |  |  | Idle/scattered trees |  |  |  |  |  |  | Disturbance? |  |  |  |  |  | NO |  | Soil Series mapped: |  |  |  |  |  | Pate |  |  |  |  |  |  | Soil Series of Boring: |  |  |  |  | Pate |  |  |  |
| SOIL PROPERTIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DEPTH | SOIL COLOR |  |  |  |  |  |  |  |  |  |  |  |  |  | TEXTURE |  | STRUCTURE |  |  |  |  | CONSIS- |  |  | ROOTS |  |  | EFFERVES- |  |  |  | PARENT |  |  |  | % |  | OTHER |  |  |  |
| (IN) | MATRIX |  |  |  |  | MOTTLES |  |  |  |  | COATINGS |  |  |  | USDA |  | USDA |  |  |  |  | TANCY |  |  |  |  |  | CENCE |  |  |  | MATERIAL |  |  |  | CLAY |  |  |  |  |  |
| 0 – 4 | 10YR 5/3 |  |  |  |  |  |  |  |  |  |  |  |  |  | Silt loam |  | Moderate medium granular |  |  |  |  | Friable |  |  | Many |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 24 |  | 3% channers |  |  |  |
| A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 – 9 | 10YR 5/4 |  |  |  |  | 10YR 6/6 |  |  |  |  | 10YR 5/3 Clay |  |  |  | Silty clay loam |  | Moderate fine and medium subangular blocky |  |  |  |  | Firm |  |  | Many |  |  |  |  |  |  | Limestone/ Colluvium |  |  |  | 30 |  | 5% channers |  |  |  |
| 2Bt1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 – 28 | 2.5Y 5/4 |  |  |  |  | 10YR 6/6 |  |  |  |  | 10YR 5/3 Clay |  |  |  | Silty clay loam |  | Weak medium subangular blocky |  |  |  |  | Firm |  |  | Many to few in the lower part |  |  |  |  |  |  | Shale residuum/ Colluvium |  |  |  | 38 |  | 8% channers |  |  |  |
| 2Bt2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 – 58+ | 2.5Y 5/4 |  |  |  |  | 10YR 6/6 |  |  |  |  | 10YR 6/1 Clay |  |  |  | Silty clay |  | Weak medium subangular blocky |  |  |  |  | Firm |  |  |  |  |  |  |  |  |  | Shale residuum |  |  |  | 56 |  | 7% channers |  |  |  |
| 3Bt3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| BORING #: |  |  |  | 5 |  |  |  |  | DESCRIPTION OF BORING LOCATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Small backhoe pit near center of the proposed lot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upland |  | Terrace |  |  |  |  |  | Outwash Plain |  |  |  |  |  | Ridge Top |  |  |  | Upper 1/3 |  |  | Middle |  |  |  |  | Lower 1/3 |  |  |  | Pothole |  |  |  | Depression |  |  | Broad Flat |  |  | Flood Plain |  |
| X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ponding or Flooding: |  |  |  |  |  |  | NO |  |  | % Slope |  |  | 7 |  |  | Aspect |  |  | N |  |  |  | Family Classification: |  |  |  |  |  | Fine, illitic, mesic Typic Hapludalfs |  |  |  |  |  |  |  |  |  |  |  |  |
| < 0.25 gpd/ft2 |  |  | > 1.20 gpd/ft2 |  |  |  |  |  |  | Seasonal High Water Table |  |  |  |  |  |  | Fragipan |  |  |  |  | Glacial Till |  |  |  |  | Bedrock |  |  |  | Massive Clay |  |  |  | Poor Filter |  |  |  | Other |  |  |
|  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  | Slickensides 48” |  |  |
| Vegetation at site: |  |  |  |  | Idle/scattered trees |  |  |  |  |  |  | Disturbance? |  |  |  |  |  | NO |  | Soil Series mapped: |  |  |  |  |  | Pate |  |  |  |  |  |  | Soil Series of Boring: |  |  |  |  | Pate |  |  |  |
| SOIL PROPERTIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DEPTH | SOIL COLOR |  |  |  |  |  |  |  |  |  |  |  |  |  | TEXTURE |  | STRUCTURE |  |  |  |  | CONSIS- |  |  | ROOTS |  |  | EFFERVES- |  |  |  | PARENT |  |  |  | % |  | OTHER |  |  |  |
| (IN) | MATRIX |  |  |  |  | MOTTLES |  |  |  |  | COATINGS |  |  |  | USDA |  | USDA |  |  |  |  | TANCY |  |  |  |  |  | CENCE |  |  |  | MATERIAL |  |  |  | CLAY |  |  |  |  |  |
| 0 – 10 | 10YR 5/3 |  |  |  |  |  |  |  |  |  |  |  |  |  | Silty clay loam |  | Moderate medium granular |  |  |  |  | Friable |  |  | Many |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 28 |  | 1% channers |  |  |  |
| A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 – 15 | 2.5Y 4/2 |  |  |  |  |  |  |  |  |  |  |  |  |  | Silty clay loam |  | Moderate medium angular blocky |  |  |  |  | Firm |  |  | Many |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 32 |  | 3% channers |  |  |  |
| A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 – 20 | 2.5Y 5/3 |  |  |  |  |  |  |  |  |  | 10YR 5/4 Clay |  |  |  | Silty clay loam |  | Moderate fine and medium subangular blocky |  |  |  |  | Firm |  |  | Many to few in the lower part |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 38 |  | 3% channers |  |  |  |
| Bt1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 – 33 | 7.5YR 4/3 |  |  |  |  |  |  |  |  |  | 10YR 4/2 Clay |  |  |  | Silty clay loam |  | Moderate medium subangular blocky |  |  |  |  | Firm |  |  | Few |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 38 |  | 3% channers |  |  |  |
| Bt3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33 – 48 | 2.5Y 5/3 |  |  |  |  |  |  |  |  |  | 10YR 4/4 Clay |  |  |  | Silty clay |  | Moderate medium subangular blocky |  |  |  |  | Firm |  |  |  |  |  |  |  |  |  | Shale residuum |  |  |  | 56 |  | 7% channers |  |  |  |
| 2Bt4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 48 – 60 | 2.5Y 5/6 |  |  |  |  | 10YR 6/6 |  |  |  |  | 10YR 6/1 Clay |  |  |  | Silty clay |  | Weak medium angular blocky |  |  |  |  | Firm |  |  |  |  |  |  |  |  |  | Shale residuum |  |  |  | 58 |  | 5% channers |  |  |  |
| 2Btss |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| BORING #: |  |  |  | 6 |  |  |  |  | DESCRIPTION OF BORING LOCATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Small backhoe pit near center of the proposed lot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upland |  | Terrace |  |  |  |  |  | Outwash Plain |  |  |  |  |  | Ridge Top |  |  |  | Upper 1/3 |  |  | Middle |  |  |  |  | Lower 1/3 |  |  |  | Pothole |  |  |  | Depression |  |  | Broad Flat |  |  | Flood Plain |  |
| X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ponding or Flooding: |  |  |  |  |  |  | NO |  |  | % Slope |  |  | 7 |  |  | Aspect |  |  | N |  |  |  | Family Classification: |  |  |  |  |  | Fine, illitic, mesic Typic Hapludalfs |  |  |  |  |  |  |  |  |  |  |  |  |
| < 0.25 gpd/ft2 |  |  | > 1.20 gpd/ft2 |  |  |  |  |  |  | Seasonal High Water Table |  |  |  |  |  |  | Fragipan |  |  |  |  | Glacial Till |  |  |  |  | Bedrock |  |  |  | Massive Clay |  |  |  | Poor Filter |  |  |  | Other |  |  |
|  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  | Slickensides 41” |  |  |
| Vegetation at site: |  |  |  |  | Idle/scattered trees |  |  |  |  |  |  | Disturbance? |  |  |  |  |  | NO |  | Soil Series mapped: |  |  |  |  |  | Pate |  |  |  |  |  |  | Soil Series of Boring: |  |  |  |  | Pate |  |  |  |
| SOIL PROPERTIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DEPTH | SOIL COLOR |  |  |  |  |  |  |  |  |  |  |  |  |  | TEXTURE |  | STRUCTURE |  |  |  |  | CONSIS- |  |  | ROOTS |  |  | EFFERVES- |  |  |  | PARENT |  |  |  | % |  | OTHER |  |  |  |
| (IN) | MATRIX |  |  |  |  | MOTTLES |  |  |  |  | COATINGS |  |  |  | USDA |  | USDA |  |  |  |  | TANCY |  |  |  |  |  | CENCE |  |  |  | MATERIAL |  |  |  | CLAY |  |  |  |  |  |
| 0 – 6 | 10YR 4/2 |  |  |  |  |  |  |  |  |  |  |  |  |  | Silt loam |  | Moderate medium granular |  |  |  |  | Friable |  |  | Many |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 25 |  | 1% channers |  |  |  |
| A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 – 11 | 2.5Y 5/1 |  |  |  |  |  |  |  |  |  |  |  |  |  | Silty clay loam |  | Moderate fine and medium subangular blocky |  |  |  |  | Firm |  |  | Many |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 32 |  | 3% channers |  |  |  |
| A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 – 41 | 2.5Y 6/4 |  |  |  |  |  |  |  |  |  | 10YR 5/4 Clay |  |  |  | Silty clay |  | Moderate medium subangular blocky |  |  |  |  | Firm |  |  | Many to few in the lower part |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 45 |  | 9% channers |  |  |  |
| Bt1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 41 – 60 | 2.5Y 5/6 |  |  |  |  | 10YR 6/6 |  |  |  |  | 10YR 6/1 Clay |  |  |  | Silty clay |  | Weak medium angular blocky |  |  |  |  | Firm |  |  |  |  |  |  |  |  |  | Shale residuum |  |  |  | 58 |  | 14% channers |  |  |  |
| 2Btss |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| BORING #: |  |  |  | 7 |  |  |  |  | DESCRIPTION OF BORING LOCATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Small backhoe pit near center of the proposed lot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upland |  | Terrace |  |  |  |  |  | Outwash Plain |  |  |  |  |  | Ridge Top |  |  |  | Upper 1/3 |  |  | Middle |  |  |  |  | Lower 1/3 |  |  |  | Pothole |  |  |  | Depression |  |  | Broad Flat |  |  | Flood Plain |  |
| X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ponding or Flooding: |  |  |  |  |  |  | NO |  |  | % Slope |  |  | 7 |  |  | Aspect |  |  | N |  |  |  | Family Classification: |  |  |  |  |  | Fine, illitic, mesic Typic Hapludalfs |  |  |  |  |  |  |  |  |  |  |  |  |
| < 0.25 gpd/ft2 |  |  | > 1.20 gpd/ft2 |  |  |  |  |  |  | Seasonal High Water Table |  |  |  |  |  |  | Fragipan |  |  |  |  | Glacial Till |  |  |  |  | Bedrock |  |  |  | Massive Clay |  |  |  | Poor Filter |  |  |  | Other |  |  |
|  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  | Slickensides 46” |  |  |
| Vegetation at site: |  |  |  |  | Idle/scattered trees |  |  |  |  |  |  | Disturbance? |  |  |  |  |  | NO |  | Soil Series mapped: |  |  |  |  |  | Pate |  |  |  |  |  |  | Soil Series of Boring: |  |  |  |  | Pate |  |  |  |
| SOIL PROPERTIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DEPTH | SOIL COLOR |  |  |  |  |  |  |  |  |  |  |  |  |  | TEXTURE |  | STRUCTURE |  |  |  |  | CONSIS- |  |  | ROOTS |  |  | EFFERVES- |  |  |  | PARENT |  |  |  | % |  | OTHER |  |  |  |
| (IN) | MATRIX |  |  |  |  | MOTTLES |  |  |  |  | COATINGS |  |  |  | USDA |  | USDA |  |  |  |  | TANCY |  |  |  |  |  | CENCE |  |  |  | MATERIAL |  |  |  | CLAY |  |  |  |  |  |
| 0 – 5 | 10YR 4/2 |  |  |  |  |  |  |  |  |  |  |  |  |  | Silty clay loam |  | Moderate medium angular blocky |  |  |  |  | Friable |  |  | Many |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 28 |  | 1% channers |  |  |  |
| A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 – 13 | 10YR 5/4 |  |  |  |  |  |  |  |  |  |  |  |  |  | Silty clay loam |  | Weak medium subangular blocky |  |  |  |  | Firm |  |  | Many |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 32 |  | 3% channers |  |  |  |
| A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 – 33 | 10YR 5/4 |  |  |  |  |  |  |  |  |  | 10YR 6/3 Clay |  |  |  | Silty clay loam |  | Moderate medium subangular blocky |  |  |  |  | Firm |  |  | Many to few in the lower part |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 36 |  | 3% channers |  |  |  |
| Bt1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33 – 46 | 10YR 5/4 |  |  |  |  |  |  |  |  |  | 10YR 6/3 Clay |  |  |  | Silty clay loam |  | Weak medium subangular blocky |  |  |  |  | Firm |  |  | Few |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 38 |  | 3% channers |  |  |  |
| Bt2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 46 – 60 | 2.5Y 5/4 |  |  |  |  | 10YR 6/6 |  |  |  |  | 10YR 6/3 Clay |  |  |  | Silty clay |  | Weak medium angular blocky |  |  |  |  | Firm |  |  |  |  |  |  |  |  |  | Shale residuum |  |  |  | 59 |  | 5% channers |  |  |  |
| 2Btss |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| BORING #: |  |  |  | 8 |  |  |  |  | DESCRIPTION OF BORING LOCATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Small backhoe pit near center of the proposed lot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upland |  | Terrace |  |  |  |  |  | Outwash Plain |  |  |  |  |  | Ridge Top |  |  |  | Upper 1/3 |  |  | Middle |  |  |  |  | Lower 1/3 |  |  |  | Pothole |  |  |  | Depression |  |  | Broad Flat |  |  | Flood Plain |  |
| X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ponding or Flooding: |  |  |  |  |  |  | NO |  |  | % Slope |  |  | 8 |  |  | Aspect |  |  | N |  |  |  | Family Classification: |  |  |  |  |  | Fine, illitic, mesic Typic Hapludalfs |  |  |  |  |  |  |  |  |  |  |  |  |
| < 0.25 gpd/ft2 |  |  | > 1.20 gpd/ft2 |  |  |  |  |  |  | Seasonal High Water Table |  |  |  |  |  |  | Fragipan |  |  |  |  | Glacial Till |  |  |  |  | Bedrock |  |  |  | Massive Clay |  |  |  | Poor Filter |  |  |  | Other |  |  |
|  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  | Slickensides 42” |  |  |
| Vegetation at site: |  |  |  |  | Idle/scattered trees |  |  |  |  |  |  | Disturbance? |  |  |  |  |  | NO |  | Soil Series mapped: |  |  |  |  |  | Pate |  |  |  |  |  |  | Soil Series of Boring: |  |  |  |  | Pate |  |  |  |
| SOIL PROPERTIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DEPTH | SOIL COLOR |  |  |  |  |  |  |  |  |  |  |  |  |  | TEXTURE |  | STRUCTURE |  |  |  |  | CONSIS- |  |  | ROOTS |  |  | EFFERVES- |  |  |  | PARENT |  |  |  | % |  | OTHER |  |  |  |
| (IN) | MATRIX |  |  |  |  | MOTTLES |  |  |  |  | COATINGS |  |  |  | USDA |  | USDA |  |  |  |  | TANCY |  |  |  |  |  | CENCE |  |  |  | MATERIAL |  |  |  | CLAY |  |  |  |  |  |
| 0 – 5 | 10YR 4/2 |  |  |  |  |  |  |  |  |  |  |  |  |  | Silt loam |  | Moderate medium granular |  |  |  |  | Friable |  |  | Many |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 25 |  | 1% channers |  |  |  |
| A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 – 16 | 7.5YR 5/4 |  |  |  |  |  |  |  |  |  |  |  |  |  | Silty clay loam |  | Moderate medium subangular blocky |  |  |  |  | Firm |  |  | Many |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 30 |  | 3% channers |  |  |  |
| A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 – 32 | 10YR 5/3 |  |  |  |  |  |  |  |  |  | 10YR 4/3 Clay |  |  |  | Silty clay loam |  | Moderate medium subangular blocky |  |  |  |  | Firm |  |  | Many to few in the lower part |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 38 |  | 3% channers |  |  |  |
| Bt1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 32 – 42 | 10YR 5/4 |  |  |  |  |  |  |  |  |  | 10YR 5/3 Clay |  |  |  | Silty clay loam |  | Weak medium subangular blocky |  |  |  |  | Firm |  |  | Few |  |  |  |  |  |  | Loess/Colluvium |  |  |  | 38 |  | 3% channers |  |  |  |
| Bt2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 42 – 60+ | 2.5Y 5/4 |  |  |  |  | 10YR 6/6 |  |  |  |  | 10YR 6/1 Clay |  |  |  | Silty clay |  | Weak medium angular blocky |  |  |  |  | Firm |  |  |  |  |  |  |  |  |  | Shale residuum |  |  |  | 60 |  | 5% channers |  |  |  |
| 2Btss |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| BORING #: |  |  |  | 9 |  |  |  |  | DESCRIPTION OF BORING LOCATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Bucket auger boring between old brick home and newer double wide |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upland |  | Terrace |  |  |  |  |  | Outwash Plain |  |  |  |  |  | Ridge Top |  |  |  | Upper 1/3 |  |  | Middle |  |  |  |  | Lower 1/3 |  |  |  | Pothole |  |  |  | Depression |  |  | Broad Flat |  |  | Flood Plain |  |
| X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ponding or Flooding: |  |  |  |  |  |  | NO |  |  | % Slope |  |  | 8 |  |  | Aspect |  |  | N |  |  |  | Family Classification: |  |  |  |  |  | Fine, illitic, mesic Typic Hapludalfs |  |  |  |  |  |  |  |  |  |  |  |  |
| < 0.25 gpd/ft2 |  |  | > 1.20 gpd/ft2 |  |  |  |  |  |  | Seasonal High Water Table |  |  |  |  |  |  | Fragipan |  |  |  |  | Glacial Till |  |  |  |  | Bedrock |  |  |  | Massive Clay |  |  |  | Poor Filter |  |  |  | Other |  |  |
|  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  | Slickensides 47” |  |  |
| Vegetation at site: |  |  |  |  | Idle/scattered trees |  |  |  |  |  |  | Disturbance? |  |  |  |  |  | NO |  | Soil Series mapped: |  |  |  |  |  | Pate |  |  |  |  |  |  | Soil Series of Boring: |  |  |  |  | unamed |  |  |  |
| SOIL PROPERTIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DEPTH | SOIL COLOR |  |  |  |  |  |  |  |  |  |  |  |  |  | TEXTURE |  | STRUCTURE |  |  |  |  | CONSIS- |  |  | ROOTS |  |  | EFFERVES- |  |  |  | PARENT |  |  |  | % |  | OTHER |  |  |  |
| (IN) | MATRIX |  |  |  |  | MOTTLES |  |  |  |  | COATINGS |  |  |  | USDA |  | USDA |  |  |  |  | TANCY |  |  |  |  |  | CENCE |  |  |  | MATERIAL |  |  |  | CLAY |  |  |  |  |  |
| 0 – 7 | 10YR 4/3 |  |  |  |  |  |  |  |  |  |  |  |  |  | Silt loam |  | Moderate medium granular |  |  |  |  | Friable |  |  | Many |  |  |  |  |  |  | Loess |  |  |  | 17 |  |  |  |  |  |
| A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 – 18 | 7.5YR 5/6 |  |  |  |  |  |  |  |  |  | 10YR 4/3 Clay |  |  |  | Very fine sandy loam |  | Moderate fine subangular blocky |  |  |  |  | Friable |  |  | Many |  |  |  |  |  |  | Windblown sand |  |  |  | 14 |  |  |  |  |  |
| 2Bt1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 – 32 | 2.5Y 5/3 |  |  |  |  |  |  |  |  |  | 10YR 4/4 Clay |  |  |  | Silty clay loam |  | Moderate medium subangular blocky |  |  |  |  | Firm |  |  | Many to few in the lower part |  |  |  |  |  |  | Limestone/ Colluvium |  |  |  | 36 |  | 3% channers |  |  |  |
| 3Bt2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 32 – 47 | 2.5Y 5/4 |  |  |  |  |  |  |  |  |  | 10YR 5/3 Clay |  |  |  | Silty clay loam |  | Weak medium subangular blocky |  |  |  |  | Firm |  |  | Few |  |  |  |  |  |  | Limestone/ Colluvium |  |  |  | 38 |  | 9% channers |  |  |  |
| 3Bt3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 47 – 60 | 2.5Y 5/4 |  |  |  |  | 10YR 6/6 |  |  |  |  | 10YR 6/1 Clay |  |  |  | Silt clay |  | Weak medium angular blocky |  |  |  |  | Firm |  |  |  |  |  |  |  |  |  | Shale residuum |  |  |  | 55 |  | 5% channers |  |  |  |
| 4Btss |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| BORING #: |  |  |  | 10 |  |  |  |  | DESCRIPTION OF BORING LOCATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Bucket auger boring behind block garage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upland |  | Terrace |  |  |  |  |  | Outwash Plain |  |  |  |  |  | Ridge Top |  |  |  | Upper 1/3 |  |  | Middle |  |  |  |  | Lower 1/3 |  |  |  | Pothole |  |  |  | Depression |  |  | Broad Flat |  |  | Flood Plain |  |
| X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ponding or Flooding: |  |  |  |  |  |  | NO |  |  | % Slope |  |  | 8 |  |  | Aspect |  |  | N |  |  |  | Family Classification: |  |  |  |  |  | Fine loamy, mixed, mesic Typic Hapludalfs |  |  |  |  |  |  |  |  |  |  |  |  |
| < 0.25 gpd/ft2 |  |  | > 1.20 gpd/ft2 |  |  |  |  |  |  | Seasonal High Water Table |  |  |  |  |  |  | Fragipan |  |  |  |  | Glacial Till |  |  |  |  | Bedrock |  |  |  | Massive Clay |  |  |  | Poor Filter |  |  |  | Other |  |  |
|  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vegetation at site: |  |  |  |  | Idle/scattered trees |  |  |  |  |  |  | Disturbance? |  |  |  |  |  | NO |  | Soil Series mapped: |  |  |  |  |  | Pate |  |  |  |  |  |  | Soil Series of Boring: |  |  |  |  | Bloomfield |  |  |  |
| SOIL PROPERTIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DEPTH | SOIL COLOR |  |  |  |  |  |  |  |  |  |  |  |  |  | TEXTURE |  | STRUCTURE |  |  |  |  | CONSIS- |  |  | ROOTS |  |  | EFFERVES- |  |  |  | PARENT |  |  |  | % |  | OTHER |  |  |  |
| (IN) | MATRIX |  |  |  |  | MOTTLES |  |  |  |  | COATINGS |  |  |  | USDA |  | USDA |  |  |  |  | TANCY |  |  |  |  |  | CENCE |  |  |  | MATERIAL |  |  |  | CLAY |  |  |  |  |  |
| 0 – 6 | 10YR 4/3 |  |  |  |  |  |  |  |  |  |  |  |  |  | Silt loam |  | Moderate medium granular |  |  |  |  | Friable |  |  | Many |  |  |  |  |  |  | Loess |  |  |  | 17 |  |  |  |  |  |
| A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 – 24 | 7.5YR 5/6 |  |  |  |  |  |  |  |  |  | 10YR 4/3 Clay |  |  |  | Very fine sandy loam |  | Moderate fine subangular blocky |  |  |  |  | Friable |  |  | Many to few in the lower part |  |  |  |  |  |  | Windblown sand |  |  |  | 22 |  |  |  |  |  |
| 2Bt1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 – 40 | 10YR 5/6 |  |  |  |  |  |  |  |  |  | 10YR 4/4 Clay |  |  |  | Very fine sandy loam |  | Weak medium subangular blocky |  |  |  |  | Friable |  |  | Few |  |  |  |  |  |  | Windblown sand |  |  |  | 15 |  |  |  |  |  |
| 2Bt2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 – 60 | 10YR 5/4 |  |  |  |  | 10YR 6/3 10YR 6/6 |  |  |  |  |  |  |  |  | Fine sandy loam |  | Weak medium subangular blocky |  |  |  |  | Friable |  |  |  |  |  |  |  |  |  | Windblown sand |  |  |  | 12 |  |  |  |  |  |
| 2BC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| BORING #: |  |  |  | 11 |  |  |  |  | DESCRIPTION OF BORING LOCATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Push probe boring on ridge top near likeliest location for home |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upland |  | Terrace |  |  |  |  |  | Outwash Plain |  |  |  |  |  | Ridge Top |  |  |  | Upper 1/3 |  |  | Middle |  |  |  |  | Lower 1/3 |  |  |  | Pothole |  |  |  | Depression |  |  | Broad Flat |  |  | Flood Plain |  |
| X |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ponding or Flooding: |  |  |  |  |  |  | NO |  |  | % Slope |  |  | 5 |  |  | Aspect |  |  | N |  |  |  | Family Classification: |  |  |  |  |  | Fine-silty over clayey, mixed, mesic Typic Hapludalfs |  |  |  |  |  |  |  |  |  |  |  |  |
| < 0.25 gpd/ft2 |  |  | > 1.20 gpd/ft2 |  |  |  |  |  |  | Seasonal High Water Table |  |  |  |  |  |  | Fragipan |  |  |  |  | Glacial Till |  |  |  |  | Bedrock |  |  |  | Massive Clay |  |  |  | Poor Filter |  |  |  | Other |  |  |
|  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  | Slickensides 37” |  |  |
| Vegetation at site: |  |  |  |  | Idle/scattered trees |  |  |  |  |  |  | Disturbance? |  |  |  |  |  | NO |  | Soil Series mapped: |  |  |  |  |  | Switzerland |  |  |  |  |  |  | Soil Series of Boring: |  |  |  |  | Switzerland |  |  |  |
| SOIL PROPERTIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DEPTH | SOIL COLOR |  |  |  |  |  |  |  |  |  |  |  |  |  | TEXTURE |  | STRUCTURE |  |  |  |  | CONSIS- |  |  | ROOTS |  |  | EFFERVES- |  |  |  | PARENT |  |  |  | % |  | OTHER |  |  |  |
| (IN) | MATRIX |  |  |  |  | MOTTLES |  |  |  |  | COATINGS |  |  |  | USDA |  | USDA |  |  |  |  | TANCY |  |  |  |  |  | CENCE |  |  |  | MATERIAL |  |  |  | CLAY |  |  |  |  |  |
| 0 – 6 | 10YR 4/4 |  |  |  |  |  |  |  |  |  |  |  |  |  | Silt loam |  | Moderate medium granular |  |  |  |  | Friable |  |  | Many |  |  |  |  |  |  | Loess |  |  |  | 22 |  |  |  |  |  |
| A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 – 12 | 7.5YR 5/6 |  |  |  |  |  |  |  |  |  | 10YR 4/4 Clay |  |  |  | Silty clay loam |  | Moderate fine subangular blocky |  |  |  |  | Firm |  |  | Many |  |  |  |  |  |  | Loess |  |  |  | 31 |  |  |  |  |  |
| Bt1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 – 26 | 7.5YR 5/6 |  |  |  |  | 10YR 6/6 |  |  |  |  | 10YR 6/3 Clay |  |  |  | Silty clay loam |  | Moderate fine and medium subangular blocky |  |  |  |  | Firm |  |  | Many to few in the lower part |  |  |  |  |  |  | Loess |  |  |  | 38 |  |  |  |  |  |
| Bt2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 – 37 | 7.5YR 5/6 |  |  |  |  |  |  |  |  |  | 10YR 6/2 Clay |  |  |  | Silty clay |  | Moderate medium angular blocky |  |  |  |  | Firm |  |  | Few |  |  |  |  |  |  | Shale residuum |  |  |  | 47 |  |  |  |  |  |
| 2Bt3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 37 – 60 | 2.5Y 6/4 |  |  |  |  | 10YR 6/6 |  |  |  |  | 10YR 6/2 Clay |  |  |  | Silty clay |  | Weak medium angular blocky |  |  |  |  | Firm |  |  |  |  |  |  |  |  |  | Shale residuum |  |  |  | 58 |  | 2% gravel |  |  |  |
| 2Btss |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| BORING #: |  |  |  | 12 |  |  |  |  | DESCRIPTION OF BORING LOCATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Push probe boring on ridge top near likeliest location for home |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upland |  | Terrace |  |  |  |  |  | Outwash Plain |  |  |  |  |  | Ridge Top |  |  |  | Upper 1/3 |  |  | Middle |  |  |  |  | Lower 1/3 |  |  |  | Pothole |  |  |  | Depression |  |  | Broad Flat |  |  | Flood Plain |  |
| X |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ponding or Flooding: |  |  |  |  |  |  | NO |  |  | % Slope |  |  | 5 |  |  | Aspect |  |  | W |  |  |  | Family Classification: |  |  |  |  |  | Fine-silty over clayey, mixed, mesic Typic Hapludalfs |  |  |  |  |  |  |  |  |  |  |  |  |
| < 0.25 gpd/ft2 |  |  | > 1.20 gpd/ft2 |  |  |  |  |  |  | Seasonal High Water Table |  |  |  |  |  |  | Fragipan |  |  |  |  | Glacial Till |  |  |  |  | Bedrock |  |  |  | Massive Clay |  |  |  | Poor Filter |  |  |  | Other |  |  |
|  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  | Slickensides 43” |  |  |
| Vegetation at site: |  |  |  |  | Idle/scattered trees |  |  |  |  |  |  | Disturbance? |  |  |  |  |  | NO |  | Soil Series mapped: |  |  |  |  |  | Switzerland |  |  |  |  |  |  | Soil Series of Boring: |  |  |  |  | Switzerland |  |  |  |
| SOIL PROPERTIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DEPTH | SOIL COLOR |  |  |  |  |  |  |  |  |  |  |  |  |  | TEXTURE |  | STRUCTURE |  |  |  |  | CONSIS- |  |  | ROOTS |  |  | EFFERVES- |  |  |  | PARENT |  |  |  | % |  | OTHER |  |  |  |
| (IN) | MATRIX |  |  |  |  | MOTTLES |  |  |  |  | COATINGS |  |  |  | USDA |  | USDA |  |  |  |  | TANCY |  |  |  |  |  | CENCE |  |  |  | MATERIAL |  |  |  | CLAY |  |  |  |  |  |
| 0 – 8 | 10YR 4/4 |  |  |  |  |  |  |  |  |  |  |  |  |  | Silt loam |  | Moderate medium granular |  |  |  |  | Friable |  |  | Many |  |  |  |  |  |  | Loess |  |  |  | 21 |  |  |  |  |  |
| A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 – 28 | 7.5YR 5/6 |  |  |  |  |  |  |  |  |  | 10YR 4/4 Clay |  |  |  | Silty clay loam |  | Moderate fine and medium subangular blocky |  |  |  |  | Firm |  |  | Many to few in the lower part |  |  |  |  |  |  | Loess |  |  |  | 33 |  |  |  |  |  |
| Bt1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 – 43 | 7.5YR 5/6 |  |  |  |  |  |  |  |  |  | 10YR 6/3 Clay |  |  |  | Silty clay |  | Moderate medium angular blocky |  |  |  |  | Firm |  |  |  |  |  |  |  |  |  | Shale residuum |  |  |  | 46 |  |  |  |  |  |
| 2Bt2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 43 – 60 | 2.5Y 6/4 |  |  |  |  | 10YR 6/6 |  |  |  |  | 10YR 6/1 Clay |  |  |  | Silty clay |  | Weak medium angular blocky |  |  |  |  | Firm |  |  |  |  |  |  |  |  |  | Shale residuum |  |  |  | 56 |  | 2% gravel |  |  |  |
| 2Btss |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| BORING #: |  |  |  | 13 |  |  |  |  | DESCRIPTION OF BORING LOCATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Push probe boring on ridge top near likeliest location for home |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upland |  | Terrace |  |  |  |  |  | Outwash Plain |  |  |  |  |  | Ridge Top |  |  |  | Upper 1/3 |  |  | Middle |  |  |  |  | Lower 1/3 |  |  |  | Pothole |  |  |  | Depression |  |  | Broad Flat |  |  | Flood Plain |  |
| X |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ponding or Flooding: |  |  |  |  |  |  | NO |  |  | % Slope |  |  | 5 |  |  | Aspect |  |  | W |  |  |  | Family Classification: |  |  |  |  |  | Fine-silty over clayey, mixed, mesic Typic Hapludalfs |  |  |  |  |  |  |  |  |  |  |  |  |
| < 0.25 gpd/ft2 |  |  | > 1.20 gpd/ft2 |  |  |  |  |  |  | Seasonal High Water Table |  |  |  |  |  |  | Fragipan |  |  |  |  | Glacial Till |  |  |  |  | Bedrock |  |  |  | Massive Clay |  |  |  | Poor Filter |  |  |  | Other |  |  |
|  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | +60” |  |  |  |  |  |  |  |  |  |  |  | Slickensides 46” |  |  |
| Vegetation at site: |  |  |  |  | Idle/scattered trees |  |  |  |  |  |  | Disturbance? |  |  |  |  |  | NO |  | Soil Series mapped: |  |  |  |  |  | Switzerland |  |  |  |  |  |  | Soil Series of Boring: |  |  |  |  | Switzerland |  |  |  |
| SOIL PROPERTIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DEPTH | SOIL COLOR |  |  |  |  |  |  |  |  |  |  |  |  |  | TEXTURE |  | STRUCTURE |  |  |  |  | CONSIS- |  |  | ROOTS |  |  | EFFERVES- |  |  |  | PARENT |  |  |  | % |  | OTHER |  |  |  |
| (IN) | MATRIX |  |  |  |  | MOTTLES |  |  |  |  | COATINGS |  |  |  | USDA |  | USDA |  |  |  |  | TANCY |  |  |  |  |  | CENCE |  |  |  | MATERIAL |  |  |  | CLAY |  |  |  |  |  |
| 0 – 7 | 10YR 4/4 |  |  |  |  |  |  |  |  |  |  |  |  |  | Silt loam |  | Moderate medium granular |  |  |  |  | Friable |  |  | Many |  |  |  |  |  |  | Loess |  |  |  | 22 |  |  |  |  |  |
| A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 – 31 | 7.5YR 5/6 |  |  |  |  |  |  |  |  |  | 10YR 4/4 Clay |  |  |  | Silty clay loam |  | Moderate fine and medium subangular blocky |  |  |  |  | Firm |  |  | Many to few in the lower part |  |  |  |  |  |  | Loess |  |  |  | 33 |  |  |  |  |  |
| Bt1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 – 46 | 7.5YR 5/6 |  |  |  |  |  |  |  |  |  | 10YR 6/3 Clay |  |  |  | Silty clay |  | Moderate medium angular blocky |  |  |  |  | Firm |  |  |  |  |  |  |  |  |  | Shale residuum |  |  |  | 45 |  | 2% gravel |  |  |  |
| 2Bt2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 46 – 60 | 2.5Y 6/4 |  |  |  |  | 10YR 6/6 |  |  |  |  | 10YR 6/2 Clay |  |  |  | Silty clay |  | Weak medium angular blocky |  |  |  |  | Firm |  |  |  |  |  |  |  |  |  | Shale residuum |  |  |  | 57 |  | 2% gravel |  |  |  |
| 2Btss |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Site Location: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

APPROXMATE

BORING LOCATIONS