Wise Texas Surveying, PLLC

Harold M. Ballard, Managing Member

RPLS No. 6329, Tx

cell:  817-999-8649

text:  940-799-9797

email:  harold.ballard@outlook.com

mailing address:

PO Box 485

Paradise, Tx     76073

www.wisetxsurveying.com

LEGAL DESCRIPTION:

RE:  JOB No.  25060101A_DOCTAVIO

BEING a 17.98 Acre Tract of land situated in BLOCK 93, VAN ZANDT COUNTY SCHOOL LAND SURVEY, ABSTRACT No. 1182, and being portion of that same tract of land described to SANJEEV KUMAR KC and NABIN THAPA recorded in DOCUMENT No. 202417434, OFFICIAL RECORDS, WISE COUNTY, TEXAS, said 17.98 Acre Tract being more particularly described by metes and bounds as follows:

COMMENCING at a Mag Nail found being the west corner of said Kumar Tract lying in the asphalt roadbed of County Road 4598, (a variable width county roadway) thence, S 86°43'46"E a of 1687.51 feet  to a the POINT OF BEGINNING of the herein described tract being east corner of said Kumar Tract from which a 1/2 Inch Capped Iron Rod found, stamped, “MDH” bears N 53°59’09” E a distance of 0.98 feet;

THENCE  S 24°59'10"W, along the northwest line of Saddlebrook Estates North, Phase 1, an addition to Wise County, Texas according to the plat recorded in Cabinet F, Slide 73, Plat Records, Wise County, Texas part of the way then along the northwest line of a tract of land described in 202307236 Official Records, Wise County, Texas a distance of 1658.87 feet  to a 1/2 Inch Capped Iron Rod, stamped, “6329” set lying along the northeast line of State Highway 114, (a variable width state right of way);

THENCE  N 32°34'56"W, along said right of way, a distance of 495.99 feet  to a 1/2 Inch Capped Iron Rod, stamped, “6329” set;

THENCE  N 12°25'04"E, along said right of way, a distance of 70.70 feet  to a Point;

THENCE  N 32°34'56"W, along said right of way, a distance of 100.00 feet  to a Point;

THENCE  N 46°36'56"W, along said right of way, a distance of 206.17 feet  to a 1/2 Inch Capped Iron Rod, stamped, “6329” set;

THENCE  N 32°34'56"W, along said right of way, a distance of 289.04 feet  to a 1/2 Inch Capped Iron Rod, stamped, “6329” set;

THENCE  N 67°21'31"E, severing said Kumar Tract, a distance of 1421.48 feet to the POINT OF BEGINNING containing 17.98 ACRES or 783351 SQUARE FEET of land.